OK bro, qo'lda yozaman — to'liq text pipeline. Har bir modul, har bir formula, nima kiradi, nima chiqadi — hammasi aniq.

## METRIC-GEE Pipeline Architecture v1.0

Platform: Google Earth Engine (Python API) Satellite: Landsat 8/9 Collection 2 Level 2 (SR + ST) Meteo: ERA5-Land hourly (GEE dagi) DEM: SRTM 30m (GEE dagi) Maqsad: Har qanday joy, har qanday vaqt — global METRIC

### MODULE 0: Configuration & Inputs

Kirish:

- Study area geometry (ee.Geometry)

- Sana oralig'i (start_date, end_date)

- Landsat scene tanlash (cloud filter < 20%)

- ERA5 vaqt oraligi (image datetime ± 1 soat)

Chiqish:

- Landsat image (SR bands + ST_B10)

- ERA5 image (soatlik, image vaqtiga mos)

- SRTM DEM image

- Image metadata (DOY, sun_elevation, acquisition_time)

Eslatma: L8 bandlari: B2(blue), B3(green), B4(red), B5(NIR), B6(SWIR1), B7(SWIR2), ST_B10(thermal). SR bandlar scale factor: 0.0000275, offset: -0.2. ST scale factor: 0.00341802, offset: 149.0.

### MODULE 1: Vegetation Indices

Kirish: SR bands (B4, B5 — scaled to reflectance)

Hisob:

- NDVI = (B5 - B4) / (B5 + B4) — F.23 modified for L8/9 bands

- SAVI_ID = (1 + L)(B5 - B4) / (L + B5 + B4) — F.19 modified, L = 0.1

- LAI = -ln((0.69 - SAVI_ID) / 0.59) / 0.91 — F.18, clamped 0–6

Chiqish: NDVI, SAVI, LAI images

Izoh: SAVI va NDVI bu yerda L2 surface reflectance ustida hisoblanadi, F.19 aslida TOA reflectance ishlatgan — bu bizning avantajimiz, aniqroq.

### MODULE 2: Albedo (Olmedo)

Kirish: SR bands (B2, B3, B4, B5, B6, B7 — scaled to reflectance)

Hisob:

- α = Olmedo formula — L8/9 SR bandlari uchun maxsus vaznlar

- F.10–14 SKIP — L2 SR allaqachon atmosferik korreksiya qilingan

- F.15 MODIFIED — Tasumi vaznlari o'rniga Olmedo vaznlari

Chiqish: Surface albedo image (α)

Izoh: Olmedo formulasi uchinchi maqoladan keyin aniqlanadi. Hozircha placeholder.

### MODULE 3: Surface Temperature (Ts)

Kirish: ST_B10 band

Hisob:

- Ts = ST_B10 × 0.00341802 + 149.0 — to'g'ridan-to'g'ri Kelvin

- F.20–21 SKIP — L2 ST allaqachon MODTRAN atmosferik korreksiya qilingan

- F.22 εNB — SKIP (L2 ST ichida hisobga olingan)

Chiqish: Ts image (Kelvin)

Izoh: L2 Surface Temperature USGS tomonidan LaSRC/TICOS orqali tuzatilgan. Rp, τNB, Rsky kerak emas.

### MODULE 4: Emissivity

Kirish: LAI, NDVI (Module 1 dan)

Hisob:

- ε₀ = 0.95 + 0.01×LAI (LAI ≤ 3), 0.98 (LAI > 3) — F.17

- εNB = 0.97 + 0.0033×LAI (LAI ≤ 3), 0.98 (LAI > 3) — F.22

- NDVI ≤ 0 (suv/qor) → ε₀ = εNB = 0.985

Chiqish: ε₀, εNB images

Izoh: εNB Module 3 da ishlatilmaydi (L2 ST), lekin RL↑ hisoblashda ε₀ kerak.

### MODULE 5: Net Radiation (Rn)

Kirish: α (Mod.2), Ts (Mod.3), ε₀ (Mod.4), ERA5 data, DEM, image metadata

Hisob:

5a. RS↓ (incoming shortwave) — F.3:

- Gsc = 1367 W/m²

- d² = F.9 (DOY dan)

- θrel = F.7 yoki F.8 (DEM slope/aspect yoki flat)

- τsw = F.4 hisoblash:

  - P = ERA5 surface_pressure / 1000 (Pa → kPa) — F.5 REPLACE

  - ea = ERA5 dewpoint → 0.6108 × exp(17.27×Td / (Td+237.3)) — F.6 REPLACE

  - W = 0.14 × ea × P + 2.1 — F.6

  - Kt = 1.0 (clean air default, yoki ERA5 aerosol dan)

  - θhor = F.8

- RS↓ = Gsc × cos(θrel) × τsw / d² — F.3

5b. RL↓ (incoming longwave) — F.24:

- εa = 0.85 × (-ln(τsw))^0.09 — F.25

- Ta = ERA5 2m_temperature (K) — meteostansiya o'rniga

- RL↓ = εa × σ × Ta⁴ — F.24

5c. RL↑ (outgoing longwave) — F.16:

- RL↑ = ε₀ × σ × Ts⁴

5d. Rn — F.2:

- Rn = RS↓ - α×RS↓ + RL↓ - RL↑ - (1-ε₀)×RL↓

Chiqish: Rn image (W/m²), τsw image

Izoh: ERA5 bilan P va ea to'g'ridan-to'g'ri olinadi, F.5 hisob formulasi DEM balandlik uchun ham ishlatilishi mumkin (backup). RL↓ da Ta uchun ERA5 t2m ishlatamiz — Ts surrogat emas.

### MODULE 6: Soil Heat Flux (G)

Kirish: Rn (Mod.5), Ts (Mod.3), NDVI (Mod.1), α (Mod.2), LAI (Mod.1)

Hisob:

- Asosiy: G/Rn = (Ts-273.15)(0.0038+0.0074α)(1-0.98×NDVI⁴) — F.26

- Alternativ (Tasumi): G/Rn = 0.05+0.18×exp(-0.521×LAI) (LAI≥0.5) — F.27a

- G/Rn = 1.80(Ts-273.15)/Rn + 0.084 (LAI<0.5) — F.27b

- Suv: G/Rn = 0.5 (NDVI<0, past albedo)

- Qor: G/Rn = 0.5 (Ts<277, NDVI<0, yuqori albedo)

- G = (G/Rn) × Rn

Chiqish: G image (W/m²)

Izoh: F.26 universal, F.27 Idaho-specific. Global model uchun F.26 default.

### MODULE 7: Wind Speed & Aerodynamics

Kirish: ERA5 (u10, v10), DEM, LAI (Mod.1), NDVI (Mod.1), α (Mod.2)

Hisob:

7a. u200 — F.32 modified:

- ERA5: ws_10 = √(u10² + v10²)

- zomw = 0.03 (o'tloq stansiya uchun default) yoki 0.015

- u200 = ws_10 × ln(200/zomw) / ln(10/zomw) — F.32 modified (zx=10m)

7b. zom (momentum roughness) — F.33 yoki F.34b:

- Option A: zom = 0.018 × LAI — F.33 (qisqa ekinlar)

- Option B: zom = exp(a1×NDVI/α + b1) — F.34b (ko'p turdagi o'simlik)

- zom_min = 0.005 m (yalang'och tuproq)

- Tog'li tuzatma: zom_mtn = zom × (1 + ((180/π)×slope - 5)/20) — F.35

7c. u (friction velocity) — F.31:*

- u* = 0.41 × u200 / ln(200/zom)

7d. rah (aerodynamic resistance, neytral) — F.30:

- rah = ln(2/0.1) / (u* × 0.41)

7e. ρair (havo zichligi) — F.37:

- P_pixel = 101.3 × ((293 - 0.0065×z)/293)^5.26 — F.5 (DEM z dan)

- ρair = 1000 × P_pixel / (1.01 × Ts × 287)

Chiqish: u200, zom, u*, rah, ρair images

Izoh: ERA5 10m shamol → 200m ga log profile bilan ko'tariladi. Tog'li sharoitda F.35-36 qo'llaniladi. Shamol tuzatma koeffitsienti ω̄ = F.36.

### MODULE 8: CIMEC Automated Calibration (Allen 2013)

Kirish: NDVI (Mod.1), Ts (Mod.3), α (Mod.2), DEM, ERA5 (precipitation, ETr), Rn (Mod.5), G (Mod.6), rah (Mod.7), ρair (Mod.7)

Bu modul METRIC ning yuragi — cold/hot pixel avtomatik tanlash!

8a. TsDEM hisoblash:

- lapse_rate = 6.5 K/km (default) yoki ERA5 profil dan

- z_datum = AOI o'rtacha balandligi

- TsDEM = Ts - lapse_rate × (z - z_datum) / 1000

8b. AOI tayyorlash:

- Agricultural mask (GEE land use dataset: USDA CDL, ESA WorldCover, yoki Dynamic World)

- Meteostansiya/ERA5 grid dan 20-30 km radius

- Cloud/shadow mask (QA_PIXEL band)

- Homogeneity filtr: NDVI CV < 15% (7×7 cluster L8 uchun)

8c. Cold pixel tanlash:

- AOI dan top 5% NDVI piksellar

- Ulardan coldest 20% TsDEM → 1% populyatsiya

- O'rtacha TsDEM hisoblash

- Filtr: TsDEM ± 0.2K va albedo ± 0.02 (F.7 dan)

- Albedo chegarasi: α = 0.001343β + 0.3281×exp(-0.0188β) — CIMEC F.7

- NDVI₁ = shu 1% populyatsiya o'rtacha NDVI

- ETrFcold = 1.05 (NDVI₁ ≥ 0.75) yoki 1.54×NDVI₁ - 0.10 — CIMEC F.4a/4b

8d. Hot pixel tanlash:

- AOI dan lowest 10% NDVI piksellar

- Ulardan hottest 20% TsDEM → 2% populyatsiya

- O'rtacha TsDEM hisoblash

- Tfac = max(0, 2.6 - 13 × P60/ETr60) — CIMEC F.8

  - P60 = ERA5 total_precipitation oxirgi 60 kun summasi

  - ETr60 = ERA5 dan ASCE PM hisob, oxirgi 60 kun summasi

- Target TsDEM = o'rtacha TsDEM - Tfac

- Filtr: TsDEM ± 0.2K

- NDVIhot = tanlangan piksel NDVI

- fc = (NDVIhot - 0.15) / (0.8 - 0.15) — CIMEC F.6

- ETrFhot = fc×ETrFcold + (1-fc)×ETrFbare — CIMEC F.5

  - ETrFbare = FAO56 tuproq suv balansidan (0.0–0.10 oralig'i)

8e. dT funksiya kalibrlash:

- Hcold = (Rn-G)cold - 1.05×ETr×λ — METRIC F.48

- Hhot = (Rn-G)hot - LEhot — METRIC F.47

- dTcold = Hcold×rah_cold / (ρair_cold×Cp) — METRIC F.49

- dThot = Hhot×rah_hot / (ρair_hot×Cp) — METRIC F.46

- a = (dThot-dTcold) / (TsDEM_hot-TsDEM_cold) — METRIC F.50

- b = (dThot-a) / TsDEM_hot — METRIC F.51

Chiqish: a, b koeffitsientlar, cold_pixel, hot_pixel, ETrFcold, ETrFhot

### MODULE 9: Sensible Heat Flux — Iterative (H)

Kirish: a, b (Mod.8), TsDEM (Mod.8), rah (Mod.7), u200 (Mod.7), zom (Mod.7), ρair (Mod.7), Rn (Mod.5), G (Mod.6), Ts (Mod.3)

Iteratsiya (har piksel uchun, 15-30 marta):

for iteration in range(30):

# 1. dT hisob

dT = a + b × TsDEM                          — F.29

# 2. H hisob

H = ρair × Cp × dT / rah                    — F.28

# 3. Monin-Obukhov uzunligi

L = -(ρair × Cp × u*³ × Ts) / (k × g × H)  — F.40

# 4. Stability correction

if L < 0 (unstable):

x200 = (1 - 16×200/L)^0.25              — F.43a

x2 = (1 - 16×2/L)^0.25                  — F.43b

x01 = (1 - 16×0.1/L)^0.25               — F.43c

ψm200 = 2ln((1+x200)/2) + ln((1+x200²)/2) - 2arctan(x200) + π/2  — F.41

ψh2 = 2ln((1+x2²)/2)                    — F.42a

ψh01 = 2ln((1+x01²)/2)                  — F.42b

elif L > 0 (stable):

ψm200 = -5×(2/L)                        — F.44

ψh2 = -5×(2/L)                          — F.45a

ψh01 = -5×(0.1/L)                       — F.45b

else:

ψm200 = ψh2 = ψh01 = 0

# 5. u* yangilash

u* = u200 × 0.41 / (ln(200/zom) - ψm200)   — F.38

# 6. rah yangilash

rah = (ln(2/0.1) - ψh2 + ψh01) / (u* × 0.41)  — F.39

# convergence check

if abs(H_new - H_old) < 0.01: break

Chiqish: H image (W/m²), final rah, final u*

Izoh: GEE da pixel-wise iteratsiya qilish uchun ee.Image.iterate() yoki 30 marta loop ishlatamiz. Convergence odatda 5-15 iteratsiyada bo'ladi.

### MODULE 10: Instantaneous ET

Kirish: Rn (Mod.5), G (Mod.6), H (Mod.9), Ts (Mod.3)

Hisob:

- LE = Rn - G - H — F.1

- λ = (2.501 - 0.00236×(Ts-273.15)) × 10⁶ — F.53

- ETinst = 3600 × LE / (λ × 1000) — F.52 (mm/hr)

Chiqish: LE, ETinst images

### MODULE 11: ETrF & Daily ET

Kirish: ETinst (Mod.10), ERA5 data, DEM

Hisob:

11a. ETr (ASCE Penman-Monteith, soatlik):

- ERA5 dan: Rn_short, T2m, u2, ea, es

- ASCE PM standart formula (alfalfa reference, Cn=1600, Cd=0.38)

- ETr = soatlik hisob, image vaqtiga mos

11b. ETrF — F.54:

- ETrF = ETinst / ETr

11c. ET24 — F.55:

- ETr24 = ERA5 dan 24 soatlik ETr summasi

- Crad = 1.0 (tekis yer) yoki F.56 (tog'li hudud)

- ET24 = Crad × ETrF × ETr24 (mm/day)

Chiqish: ETrF, ET24 images

### MODULE 12: Temporal Interpolation (Seasonal ET)

Kirish: ETrF images (bir nechta sana), ERA5 kunlik ETr

Hisob:

- ETrF interpolatsiya: linear yoki cubic spline (sanalar orasida)

- ETperiod = Σ(ETrFi × ETr24i) — F.58

- ETrF_period = Σ(ETrFi × ETr24i) / Σ(ETr24i) — F.59

Chiqish: Monthly ET, Seasonal ET, Annual ET images (mm)

### MODULE 13: Validation & Export

Kirish: ET24 yoki ETperiod (Mod.11/12)

Hisob:

- OpenET, MODIS ET, lysimeter yoki EC flux tower bilan solishtirish

- R², RMSE, MAE, bias hisoblash

- Zonal statistics (field-level, district-level)

Chiqish: Validation metrics, GeoTIFF export, CSV statistics

### FORMULA XARITASI (qaysi formula qaysi modulda)

Module 0:  —

Module 1:  F.23, F.19, F.18

Module 2:  Olmedo (F.15 modified)

Module 3:  L2 ST direct (F.20-21 SKIP)

Module 4:  F.17, F.22

Module 5:  F.2, F.3, F.4, F.5→ERA5, F.6→ERA5, F.7, F.8, F.9, F.16, F.24, F.25

Module 6:  F.26 (yoki F.27a/b)

Module 7:  F.30, F.31, F.32→ERA5, F.33/F.34b, F.35, F.36, F.37

Module 8:  CIMEC F.1-8, METRIC F.46-51

Module 9:  F.28, F.29, F.38, F.39, F.40, F.41-45

Module 10: F.1, F.52, F.53

Module 11: F.54, F.55, F.56, F.57

Module 12: F.58, F.59

Module 13: —

### SKIP qilingan formulalar (jami 7 ta)

F.10  — ρt,b TOA reflectance → L2 SR bor

F.11  — ρs,b atmospheric correction → L2 SR bor

F.12  — τin,b → kerak emas

F.13  — τout,b → kerak emas

F.14  — ρa,b path reflectance → kerak emas

F.20  — Ts Planck → L2 ST bor

F.21  — Rc thermal correction → L2 ST bor

Shu pipeline to'liq, bro. 14 ta modul, 59+8 = 67 formula, L8/9 + ERA5 + GEE uchun moslashtirilgan.

Keyingi qadam — Olmedo albedo formulasini aniqlashtirish va keyin kod yozishni boshlash. Qaysi moduldan boshlaymiz?